Date:	31 May, 2000

Dear CLG Members & HAFFA,

Advisory Guidance on DG incidents

The Research and Special Program Administration (RSPA) of DOT has published an Advisory Guidance on DG incidents related to “Batteries”. I attach the document for your information and reference.

In summary, the RSPA has recorded several incidents that “heat” generated by batteries or devices that contain batteries have caused smoke and the initiation of a fire while the device was being transported.

One significant item is the “Batteries, dry” or the generally use batteries, when not properly packed against short-circuited, when come into contact with metallic items, the temperature of the device or battery can quickly rise to a point that causes leakage of the battery or ignites nearby combustibles.

The IATA DGR stipulates:

“ Batteries, dry are not subject to the requirements of

DGR only when they are offered for transportation

in a manner that prevents the dangerous evolution of heat

(for example, by the effective insulation of exposed

terminals).”

“Batteries, dry” that are not otherwise subject to the requirements of the HMR or DGR are batteries such as rechargeable camera, cell phone, and dry carbon and alkaline batteries which are commonly used by consumers. These batteries are excepted from requirements of the HMR OR DGR when offered for transportation or transported in commerce provided the battery is packaged in a manner that prevents the generation of a dangerous quantity of heat that may result from short-circuiting.

Following is some important points reminded by RSPA:

“Any persons who offers or transports an battery or an electrical device with an installed battery, including power sources, lights or torches, power tools, and other related articles are encouraged to carefully review this guidance to examine all of their procedures, and where necessary, to take measures to prevent potential incidents in transportation. While evaluating whether such devices are likely to produce  sparks or generate a dangerous quantity of heat, environmental conditions normally encountered in transportation must be taken into account, including temperature, humidity, vibration, impacts from rough handling and other relevant factors. In addition, the possibility of product manufacturing variations such as contamination, spacings, and loose parts should be taken into account.

Persons are reminded that the offering for transportation of any forbidden material in violation of the HMR or DGR subjects the offerer to enforcement action, including, but not limited to, significant civil penalties and appropriate judicial remedies. Furthermore, a willful violation of the HMR, or the reckless offering of a material for transportation in violation of the HMR, is subject to criminal penalties.”

In view of the seriousness and up trend of Batteries incidents, I suggest to member airlines that a random inspection on the proper packing of “Batteries, dry” to be conducted. Though I have pointed out during the CLG meeting that shipper or its agent is responsible to properly classify, pack its shipment for transport, for carrier’s own safety, a verification process in accepting “Batteries” shipments should be built in to ensure compliance.

Charles Tse

CLG DG Task Force.

Attachment:

DEPARTMENT OF TRANSPORTATION

Research and Special Programs Administration

49 CFR Parts 171-180

Docket No. RSPA-99-5143

[Notice No. 99-8]

Advisory Guidance; Transportation of Batteries and Devices

That Contain Batteries

AGENCY: Research and Special Programs Administration (RSPA),

DOT.

ACTION: Advisory Guidance.

SUMMARY: RSPA has become aware of several incidents that

recently occurred where heat generated by batteries or

devices that contain batteries have caused smoke and/or the

initiation of a fire while the device or article was being

transported in commerce. This suggests that some persons

engaged in the offering of batteries and such devices for

transportation may not be fully aware of the requirements and

prohibitions of 49 CFR Parts 171-180 applicable to such

devices. This advisory guidance is to remind anyone offering

for transportation or transporting such devices that

electrical storage devices or articles that contain batteries

are forbidden from transportation unless properly packaged as

to be protected from such an occurrence.

FOR FURTHER INFORMATION CONTACT: Eric Nelson, Office of

Hazardous Materials Standards, RSPA, Department of

Transportation, 400 Seventh Street, SW., Washington, DC

20590-0001, Telephone (202) 366-8553.

SUPPLEMENTARY INFORMATION: RSPA has been made aware of

several incidents in which batteries or other devices that

contain batteries have short-circuited or otherwise

functioned in such a manner so as to generate heat, smoke, or

initiate a fire while being transported in commerce. This

advisory guidance is intended to remind persons offering for

transportation, or personally transporting any battery or

electrical device of their responsibility under the Hazardous

Materials Regulations (HMR) that any battery or electrical

device that could create sparks or generate heat may only be

offered for transportation or transported when adequately

protected from such an occurrence.

I.	Background

In May, 1994, while being delivered to a handling agent by

road, a shipment of small lithium batteries destined for

Gatwick airport in London, England, was found emitting smoke

from a Unit Loading Device. The shipment consisted of

batteries, approximately the size of a dime and about 5mm

high, which had been tossed loosely in a box. The batteries

apparently short-circuited when exposed battery terminal tabs

came into contact with other batteries, and subsequently

started a fire that significantly damaged the shipment. The

UK Civil Aviation Authorities investigated the incident. The

shipper was fined ?200 with ?00 additional costs being paid.

In February, 1996, 106 packaged lawnmowers with an electrical

battery installed were offered to an air carrier for

transportation. While in an air cargo facility, and after

being transported on two separate flights, smoke was

discovered coming from one of the boxes. Air cargo personnel

determined that an installed battery was dislodged and

short-circuited, causing the wiring, plastic housing, and

battery to burn and melt. The air carrier immediately took

action to locate the other packages, which were in the

process of being transported to other destinations throughout

the United States. The air carrier returned three airborne

flights and two taxiing aircraft to the airport, and held 11

flights preparing to depart until all 106 packages were

accounted for. Approximately 50 of the 106 lawnmower

batteries short-circuited, and several burned sufficiently to

char the packaging in which they were being shipped.

A November, 1997 incident involved a fiberboard box packaging

containing non-spillable, wet electric storage batteries

offered to an air carrier for transportation. The shipment

was picked up by a messenger service for delivery to the air

carrier s cargo facility. The package was discovered burning

prior to air transport, the probable cause of which was

short-circuiting of the battery caused by storage of cables

directly on top of the battery. The short-circuit generated

enough heat to ignite nearby combustible materials.

RSPA has become aware of several other occurrences of

passenger baggage that have been discovered smoldering or

burning as a result of battery short-circuits. These

batteries include camcorder, camera, or other dry-cell

general use batteries that have short-circuited because of

coming into contact with keys or other metallic items packed

in proximity to the batteries. When such a short-circuit

occurs, the temperature of the device or battery can quickly

rise to a point that causes leakage of the battery or ignites

nearby combustibles such as packaging materials or suitcase

linings.

Based on these and other reports, RSPA is concerned that some

persons offering for transportation or transporting batteries

or devices that contain batteries may not be fully aware of

the applicable provisions of the Hazardous Materials

Regulations (HMR; 49 CFR Parts 171-180). During air

transport, leakage from batteries, smoke and/or fire in cargo

compartments can pose a grave risk to transportation safety.

The serious potential risks posed to flight safety by

batteries and devices that contain batteries, in particular

where the battery is not otherwise subject to regulation, is

specifically addressed by RSPA and the International Civil

Aviation Organization (ICAO). On March 5, 1999, RSPA

published in the Federal Register (64 FR 10742) a final rule

in Docket No. RSPA-98-4185 (HM-215C) that amended the

Hazardous Material Table to add, among other things, special

provision 130 to the entry, Battery, dry, not subject to the

requirements of this subchapter. This special provision,

codified at ?nbsp;172.102, identifies conditions that must be

met before dry batteries that are not otherwise subject to

the HMR may be offered for transportation or transported in

commerce. It reads as follows:

“130. Batteries, dry are not subject to the requirements of

this subchapter only when they are offered for transportation

in a manner that prevents the dangerous evolution of heat

(for example, by the effective insulation of exposed

terminals).”

In addition, ?173.21 © specifies that electrical devices

that are likely to create sparks or generate a dangerous

quantity of heat, unless packaged in a manner that precludes

such an occurrence, may not be offered for transportation in

commerce.

In the 1999/2000 edition of the ICAO Technical Instructions

for the Safe Transportation of Dangerous Goods, the following

prohibitive statement appears in relation to electric storage

batteries not listed in the Dangerous Goods List (Table 2-14)

or otherwise subject to the provisions of, the Technical

Instructions:

“Any electrical battery or battery-powered device having the

potential of dangerous evolution of heat that is not prepared

so as to prevent a short circuit (e.g., in the case of

batteries, by the effective insulation of exposed terminals;

or, in the case of equipment, by disconnection of the battery

and protection of exposed terminals) is forbidden from

transport.” (Special Provision A123, pg. 2-12-8)

II.	Requirements for the transportation of electrical devices

Where the HMR permit batteries to be offered for

transportation, either separately or installed in equipment,

batteries must be protected against short circuits. The

following discusses types of batteries which are regulated by

the HMR. Wet batteries: Batteries, wet, filled with acid, UN

2794, and Batteries, wet, filled with alkali, UN 2795, are

those that contain corrosive battery fluid, and are subject

to the packing requirements of ?nbsp;173.159. Wet

“non-spillable” batteries: Batteries, wet, nonspillable, UN

2800, are wet batteries which are capable of withstanding

leakage of battery fluid when subjected to vibration and

pressure differential tests, as specified in ?nbsp;173.159

(d) (3). These batteries are excepted from all other

requirements of the HMR, provided they meet the requirements

of ?nbsp;173.159 (d) (1), (d) (2), and (d) (3). Batteries

containing sodium: Batteries, containing sodium, UN 3292, are

batteries that contain a material that, by contact with

water, are liable to become spontaneously flammable or give

off flammable or toxic gas at a rate greater than 1 liter per

kilogram of the material per hour. These batteries are

subject to the packaging requirements of ?nbsp;173.189.

Lithium batteries: Lithium batteries, UN 3090, and Lithium

batteries, contained in equipment, UN 3091, are batteries

which contain lithium substances that react dangerously with

water. Regulated batteries contain, for liquid cathodes, more

than 0.5 grams of lithium per cell, or containing an

aggregate of over 1.0 gram of lithium or lithium alloy, and

batteries which contain solid cathodes, 1.0 gram of lithium

or lithium alloy per cell, or an aggregate of over 2.0 grams

of lithium or lithium alloy. These batteries are subject to

the packaging requirements of ?nbsp;173.185. Certain lithium

batteries are not subject to the requirements of the HMR

provided they meet the requirements of ?nbsp;173.185 ©.

Batteries containing potassium hydroxide solids: Batteries,

dry, containing potassium hydroxide solid, UN 3028, are those

which contain corrosive solids, and are subject to the

packaging requirements of ?nbsp;173.213.

Dry batteries that are not otherwise subject to the

requirements of the HMR are batteries such as rechargeable

camera, cell phone, and dry carbon and alkaline batteries

which are commonly used by consumers. These batteries are

otherwise excepted from requirements of the HMR when offered

for transportation or transported in commerce provided the

battery is packaged in a manner that prevents the generation

of a dangerous quantity of heat that may result from

short-circuiting. For the purpose of ?nbsp;173.21 ©,

“dangerous quantity of heat” is considered, in part, to be a

sufficient amount of energy to cause leakage of the battery

contents, smoke or fire, or personal injury.

Even without a short-circuit condition existing, a component

in circuitry connected to a battery may become heated to a

point where combustion is initiated in the component itself,

or in near-by combustible materials, even if the battery or

the device in which the battery is installed functions

normally. RSPA has become aware of several incidents in which

devices that contain batteries, although shipped in

compliance with ?nbsp;173.159, have produced dangerous

quantities of heat while in transportation in commerce. RSPA

is evaluating the conditions surrounding these incidents.

An example is a November 1997 incident in which a device

known commonly as an Uninterruptible Power Source was offered

to an air carrier for transportation in commerce. An

Uninterruptible Power Source, a device consisting of a

battery with associated circuitry, is used both to provide

electrical surge protection to computers and to supply

emergency power to computers in the event of a loss of normal

power. After being transported on at least one flight, the

power source, packed in a fiberboard box, was discovered

burning and smoking at a cargo sort facility. A subsequent

investigation revealed that the burning initiated in a

printed circuit board, with the source of energy being a

battery within the device.

Another example of a condition of a component in circuitry

connected to a battery may become heated to a point where

combustion is initiated in the component itself, or in

near-by combustible materials, even when the device functions

normally is a properly operating, high-intensity flashlight

used by scuba divers. Such devices, packed in checked baggage

and unintentionally activated (i.e., by movement of the

on/off switch to the “on” position), have started fires in

passenger baggage.

Airline passengers and persons who offer such electrical

devices for transportation as carry on baggage, checked

baggage, or as cargo, are responsible for assuring that

appropriate means are taken to protect against dangerous

levels of heat from inadvertent activation or short-circuit

of the electrical device in transportation. Individuals who

carry any battery-powered electrical device in their luggage

should take care not to pack it in a manner that may lead to

a short-circuit by contact with keys or other metallic

articles, or its inadvertent activation while in

transportation. To address this potential risk, the HMR

contains an overriding provision in ?nbsp;173.21, Forbidden

materials and packages. Materials forbidden by ?nbsp;173.21

may not be offered for transportation, or transported in

commerce. This section extends the forbidden designation

beyond materials specifically identified in the Hazardous

Materials Table or elsewhere in the HMR, to various

additional general categories including:

“Electrical devices which are likely to create sparks or

generate a dangerous quantity of heat, unless packaged in a

manner which precludes such an occurrence.”

Any electrical device, even one not otherwise subject to the

HMR (either by specific exception from the HMR, or because

the device and its power source contains no material meeting

the definition of a hazardous material), is forbidden from

being offered for transportation, or transported, if the

device is likely to produce sparks or a dangerous quantity of

heat.

III.	Reminder to offerers and transporters

Any persons who offers or transports an battery or an

electrical device with an installed battery, including power

sources, lights or torches, power tools, and other related

articles are encouraged to carefully review this guidance, to

examine all of their procedures, and where necessary, to take

measures to prevent potential incidents in transportation.

While evaluating whether such devices are likely to produce

sparks or generate a dangerous quantity of heat,

environmental conditions normally encountered in

transportation must be taken into account, including

temperature, humidity, vibration, impacts from rough handling

and other relevant factors. In addition, the possibility of

product manufacturing variations such as contamination,

spacings, and loose parts should be taken into account.

Persons are reminded that the offering for transportation of

any forbidden material in violation of the HMR subjects the

offerer to enforcement action, including, but not limited to,

significant civil penalties and appropriate judicial

remedies. Furthermore, a willful violation of the HMR, or the

reckless offering of a material for transportation in

violation of the HMR, is subject to criminal penalties of up

to 5 years in prison and/or fines.

Issued in Washington, DC, on

Alan I. Roberts

Associate Administrator for Hazardous Materials Safety